Multi-objective Memetic Algorithm with Reinforcement Learning for Flexible Job Shop Scheduling Problem Considering Mobile Robot Cooperation

(Supplementary Materials)

# S-Ⅰ. Problem Description, Mathematical Model, and Encoding-decoding Scheme

## A. Algorithm S-1

Algorithm S-1 Forward scheduling

Input: = {r ¦ rR} Initialize: si,j = 0, ei,j = 0,oi,jO; k = 0; kr = 1, rR; - Determine βi,j,g,u for each oi,j based on its assigned group g and mode u; - while k < ¦O¦ - for each rR - Let oi,j be the kr-th operation in r; - Let = {oi′,j′ is predecessor of oi,j in r′ ¦ r′R, oi,jr′}; - if ∄oi′,j′: si′,j′ = ei′,j′ - if j = 1 - si,j = max{0, ei′,j′ ¦ oi′,j′}; ei,j = si,j+βi,j,g,u; - else - si,j = max{ei,j-1, ei′,j′ ¦ oi′,j′}; ei,j = si,j+βi,j,g,u; - end if - k = k+1; kr′ = kr′+1, r′R, oi,jr′; - end if - end for - end while Output: si,j, ei,j,oi,jO

## B. Table S-Ⅰ

TABLE S-Ⅰ

Task-Robot Group Association Information

| Task | Operation | avi,j,g |  |  |  |  | {[αi,j,g,u, βi,j,g,u] ¦ uV(i,j,g)} |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | g1 | g2 | g3 | g4 |  | g1 | g2 | g3 | g4 |
| t1 | o1,1 | 0 | 1 | 0 | 1 |  | - | {[1, 12]} | - | {[1, 10]} |
|  | o1,2 | 0 | 1 | 1 | 0 |  | - | {[1, 7]} | {[1, 8]} | - |
|  | o1,3 | 0 | 0 | 1 | 1 |  | - | - | {[1, 9]} | {[1, 10]} |
|  | o1,4 | 1 | 0 | 0 | 0 |  | {[1, 10], [2, 5]} | - | - | - |
| t2 | o2,1 | 0 | 1 | 1 | 1 |  | - | {[1, 12]} | {[1, 9]} | {[1, 10]} |
|  | o2,2 | 1 | 0 | 0 | 0 |  | {[1, 13], [2, 7], [3, 5]} | - | - | - |
|  | o2,3 | 0 | 1 | 1 | 1 |  | - | {[1, 8]} | {[1, 9]} | {[1, 10]} |
|  | o2,4 | 0 | 0 | 0 | 1 |  | - | - | - | {[1, 6]} |
| t3 | o3,1 | 0 | 1 | 1 | 1 |  | - | {[1, 7]} | {[1, 10]} | {[1, 7]} |
|  | o3,2 | 0 | 0 | 1 | 1 |  | - | - | {[1, 6]} | {[1, 8]} |
|  | o3,3 | 0 | 1 | 0 | 1 |  | - | {[1, 12}} | - | {[1, 10]} |
|  | o3,4 | 1 | 0 | 0 | 0 |  | {[2, 6], [3, 5]} | - | - | - |

## C. Table S-Ⅱ

TABLE S-Ⅱ

Start Time and End Time of Each Operation

| Operation | o1,1 | o1,2 | o1,3 | o1,4 | o2,1 | o2,2 | o2,3 | o2,4 | o3,1 | o3,2 | o3,3 | o3,4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| si,j | 0 | 10 | 17 | 26 | 0 | 9 | 17 | 25 | 0 | 9 | 15 | 25 |
| ei,j | 10 | 17 | 26 | 36 | 9 | 14 | 25 | 31 | 7 | 15 | 25 | 31 |

# S-Ⅱ. Multi-objective Memetic Algorithm with Reinforcement Learning

## A. Algorithm S-2

Algorithm S-2 Energy-saving strategy

Input: Ω - for each CRΩ - Obtain = {r ¦ rR} and Cmax() = {br ¦ rR} of CR; - si,j = Cmax(), ei,j = Cmax(),oi,jO; k = 0; kr = ¦r¦, rR; - Determine βi,j,g,u for each oi,j based on its assigned group g and mode u; - while k < ¦O¦ - for each rR - Let oi,j be the kr-th operation in r; - Let = {oi′,j′ is successor of oi,j in r′ ¦ r′R, oi,jr′}; - if ∄oi′,j′: si′,j′ = ei′,j′ - if j = ¦r¦ - ei,j = min{br, si′,j′ ¦ oi′,j′}; si,j = ei,j-βi,j,g,u; - else - ei,j = min{si,j+1, si′,j′ ¦ oi′,j′}; si,j = ei,j-βi,j,g,u; - end if - k = k+1; kr′ = kr′-1, r′R, oi,jr′; - end if - end for - end while - Update TEC() of CR; - end for - Identify the Pareto set Ω1 of Ω Output: Ω1

# S-Ⅲ. Experimental Tests and Comparisons

## A. Performance Indicator

The coverage indicator and the HV indicator are detailed as follows.

The coverage indicator, or C-metric, C(Ω′, Ω), quantifies the coverage of Pareto set Ω by Ω′ and is computed using equation (S-1). Here, ′   means that solution ′ dominates or equals  in objective values, and |Ω| indicates the number of solutions in Ω.

(S-1)

The HV indicator, or S-metric, HV(Ω, ref), is a unary metric that measures the objective space covered by a Pareto set Ω and is computed using equation (S-2). Here, δ is the Lebesgue measure, 𝒱(, ref) denotes the HV contributed by a non-dominated solution  relative to the reference point ref. The worst and best objective values are used to normalize all solutions obtained during the iterations of all algorithms in this study, and ref = (1, 1). A higher HV indicates better convergence and greater diversity of the Pareto set.

(S-2)

## B. Instance Setting

The instance generation procedure is as follows. Specifically, the instances differ in three aspects: the number of tasks n{30, 50, 100}, the number of robots M{10, 12}, and the multi-robot group configuration, including one three-robot group, one four-robot group, or two independent three-robot groups. For all instances, the number of operations per task is randomly selected from {5, 6, 10}. Among all operations, 25% must be processed by their corresponding multi-robot group, for which the processing modes and robot requirements are randomly generated. The processing time of these operations is obtained by uniformly sampling a base value from [5, 15] and dividing it by the sum of the robot requirement and a random perturbation, thereby ensuring that the processing time decreases monotonically as the number of participating robots increases. The remaining 75% of operations are processed by single-robot groups, with at most three candidate groups per operation, and their processing times are uniformly sampled from [5, 15]. In addition, the energy consumption parameters eor and eir are randomly selected from [15, 30] and [1, 5], respectively, and robots within the same group share identical parameters.

## C. Tables S-Ⅲ to S-Ⅵ

For each parameter combination, each comparison algorithm runs 10 times, and the average HV results of 10 runs are recorded in the tables, denoted as AVG, and the best AVG result for each comparison algorithm is marked in bold.

TABLE S-Ⅲ

Orthogonal Table L9(33) and the HV Results of NSGA-II

| Experiment number | Pop | pc | pm | AVG |
| --- | --- | --- | --- | --- |
| 1 | 20 | 0.2 | 0.2 | 0.95 |
| 2 | 20 | 0.5 | 0.5 | 0.96 |
| 3 | 20 | 0.8 | 0.8 | 0.96 |
| 4 | 50 | 0.2 | 0.5 | 0.95 |
| 5 | 50 | 0.5 | 0.8 | 0.94 |
| 6 | 50 | 0.8 | 0.2 | 0.91 |
| 7 | 100 | 0.2 | 0.8 | 0.91 |
| 8 | 100 | 0.5 | 0.2 | 0.90 |
| 9 | 100 | 0.8 | 0.5 | 0.91 |

TABLE S-Ⅳ

Orthogonal Table L9(33) and the HV Results of MOEA/D

| Experiment number | Pop | AR | T | AVG |
| --- | --- | --- | --- | --- |
| 1 | 20 | 20 | 2 | 0.92 |
| 2 | 20 | 50 | 5 | 0.95 |
| 3 | 20 | 100 | 10 | 0.94 |
| 4 | 50 | 20 | 5 | 0.95 |
| 5 | 50 | 50 | 10 | 0.94 |
| 6 | 50 | 100 | 2 | 0.88 |
| 7 | 100 | 20 | 10 | 0.93 |
| 8 | 100 | 50 | 2 | 0.82 |
| 9 | 100 | 100 | 5 | 0.92 |

TABLE S-Ⅴ

Orthogonal Table L9(34) and the HV Results of MO-HHMA

| Experiment number | Pop | ζ | pLS | Iter | AVG |
| --- | --- | --- | --- | --- | --- |
| 1 | 20 | 5 | 1/5 | 100 | 0.95 |
| 2 | 20 | 10 | 1/10 | 150 | 0.95 |
| 3 | 20 | 20 | 1/20 | 200 | 0.94 |
| 4 | 50 | 5 | 1/10 | 200 | 0.94 |
| 5 | 50 | 10 | 1/20 | 100 | 0.91 |
| 6 | 50 | 20 | 1/5 | 150 | 0.93 |
| 7 | 100 | 5 | 1/20 | 150 | 0.87 |
| 8 | 100 | 10 | 1/5 | 200 | 0.92 |
| 9 | 100 | 20 | 1/10 | 100 | 0.89 |

TABLE S-Ⅵ

Orthogonal Table L9(35) and the HV Results of APHMA

| Experiment number | Pop | pc | pm | η | σ | AVG |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 0.2 | 0.2 | 5 | 0.3 | 0.91 |
| 2 | 20 | 0.5 | 0.5 | 10 | 0.5 | 0.91 |
| 3 | 20 | 0.8 | 0.8 | 20 | 0.7 | 0.89 |
| 4 | 50 | 0.2 | 0.5 | 20 | 0.7 | 0.61 |
| 5 | 50 | 0.5 | 0.8 | 5 | 0.5 | 0.74 |
| 6 | 50 | 0.8 | 0.2 | 10 | 0.3 | 0.75 |
| 7 | 100 | 0.2 | 0.8 | 10 | 0.3 | 0.48 |
| 8 | 100 | 0.5 | 0.2 | 20 | 0.7 | 0.41 |
| 9 | 100 | 0.8 | 0.5 | 5 | 0.5 | 0.47 |

## D. Tables S-Ⅶ to S-Ⅸ

Tables S-Ⅶ to S-Ⅸ present the HV results of the different algorithms on the test instances. Each algorithm is independently run 10 times on each instance. The best, average, and variance of the HV values over the 10 runs are represented by BST, AVG, and VAR, respectively. Bold text in the tables indicates the best BST, AVG, and VAR for each instance.

TABLE S-Ⅶ

BST Results of Different Compared Algorithms

| Instance | MO-MARL1 | MO-MARL2 | MO-MARL | NSGA-II | MOEA/D | MO-HHMA | APHMA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30_6 | 0.95 | 0.97 | 0.98 | 0.93 | 0.96 | 0.95 | 0.75 |
| 30_7 | 0.96 | 0.96 | 0.98 | 0.96 | 0.95 | 0.96 | 0.79 |
| 30_8 | 0.98 | 0.97 | 1.00 | 0.96 | 0.96 | 0.97 | 0.81 |
| 30_9 | 0.94 | 0.97 | 0.96 | 0.93 | 0.91 | 0.94 | 0.79 |
| 30_10 | 0.94 | 0.97 | 0.97 | 0.94 | 0.93 | 0.96 | 0.84 |
| 30_11 | 0.98 | 0.97 | 0.99 | 0.96 | 0.97 | 0.97 | 0.85 |
| 50_6 | 0.96 | 0.99 | 0.99 | 0.94 | 0.97 | 0.94 | 0.82 |
| 50_7 | 0.95 | 0.99 | 0.98 | 0.93 | 0.96 | 0.93 | 0.73 |
| 50_8 | 0.97 | 0.98 | 1.00 | 0.98 | 0.98 | 0.96 | 0.83 |
| 50_9 | 0.91 | 0.96 | 0.97 | 0.91 | 0.89 | 0.91 | 0.74 |
| 50_10 | 0.92 | 0.98 | 0.98 | 0.93 | 0.94 | 0.93 | 0.78 |
| 50_11 | 0.97 | 0.99 | 1.00 | 0.96 | 0.97 | 0.96 | 0.76 |
| 100_6 | 0.89 | 0.98 | 0.99 | 0.89 | 0.95 | 0.91 | 0.66 |
| 100_7 | 0.89 | 0.97 | 0.98 | 0.92 | 0.93 | 0.91 | 0.69 |
| 100_8 | 0.93 | 0.97 | 0.99 | 0.93 | 0.92 | 0.94 | 0.69 |
| 100_9 | 0.90 | 0.96 | 0.98 | 0.89 | 0.92 | 0.89 | 0.67 |
| 100_10 | 0.89 | 0.97 | 0.98 | 0.89 | 0.89 | 0.92 | 0.65 |
| 100_11 | 0.94 | 0.98 | 0.98 | 0.94 | 0.97 | 0.92 | 0.66 |

TABLE S-Ⅷ

AVG Results of Different Compared Algorithms

| Instance | MO-MARL1 | MO-MARL2 | MO-MARL | NSGA-II | MOEA/D | MO-HHMA | APHMA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30_6 | 0.91 | 0.94 | 0.96 | 0.91 | 0.92 | 0.92 | 0.71 |
| 30_7 | 0.93 | 0.95 | 0.96 | 0.94 | 0.93 | 0.93 | 0.74 |
| 30_8 | 0.94 | 0.96 | 0.97 | 0.94 | 0.94 | 0.94 | 0.78 |
| 30_9 | 0.90 | 0.93 | 0.95 | 0.90 | 0.90 | 0.90 | 0.76 |
| 30_10 | 0.92 | 0.95 | 0.97 | 0.91 | 0.91 | 0.93 | 0.80 |
| 30_11 | 0.94 | 0.95 | 0.97 | 0.94 | 0.96 | 0.94 | 0.79 |
| 50_6 | 0.92 | 0.97 | 0.97 | 0.91 | 0.95 | 0.92 | 0.78 |
| 50_7 | 0.92 | 0.94 | 0.95 | 0.89 | 0.93 | 0.91 | 0.67 |
| 50_8 | 0.95 | 0.96 | 0.97 | 0.96 | 0.96 | 0.93 | 0.79 |
| 50_9 | 0.87 | 0.93 | 0.94 | 0.86 | 0.86 | 0.88 | 0.71 |
| 50_10 | 0.89 | 0.95 | 0.96 | 0.90 | 0.92 | 0.91 | 0.73 |
| 50_11 | 0.94 | 0.96 | 0.97 | 0.94 | 0.95 | 0.91 | 0.73 |
| 100_6 | 0.86 | 0.96 | 0.96 | 0.85 | 0.91 | 0.87 | 0.63 |
| 100_7 | 0.87 | 0.96 | 0.96 | 0.89 | 0.90 | 0.90 | 0.66 |
| 100_8 | 0.90 | 0.95 | 0.98 | 0.91 | 0.91 | 0.92 | 0.64 |
| 100_9 | 0.86 | 0.95 | 0.96 | 0.86 | 0.88 | 0.87 | 0.62 |
| 100_10 | 0.85 | 0.94 | 0.95 | 0.87 | 0.87 | 0.88 | 0.63 |
| 100_11 | 0.92 | 0.96 | 0.97 | 0.92 | 0.94 | 0.90 | 0.63 |

TABLE S-Ⅸ

VAR Results of Different Compared Algorithms

| Instance | MO-MARL1 | MO-MARL2 | MO-MARL | NSGA-II | MOEA/D | MO-HHMA | APHMA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30_6 | 9.66E-04 | 2.92E-04 | 3.12E-04 | 3.03E-04 | 3.52E-04 | 4.66E-04 | 8.72E-04 |
| 30_7 | 5.69E-04 | 1.71E-04 | 1.66E-04 | 5.14E-04 | 1.73E-04 | 2.42E-04 | 1.36E-03 |
| 30_8 | 4.26E-04 | 1.32E-04 | 2.10E-04 | 4.20E-04 | 3.40E-04 | 3.07E-04 | 9.08E-04 |
| 30_9 | 1.21E-03 | 4.55E-04 | 1.62E-04 | 4.91E-04 | 1.20E-04 | 5.81E-04 | 9.42E-04 |
| 30_10 | 1.63E-04 | 2.59E-04 | 4.12E-05 | 4.07E-04 | 1.25E-04 | 1.87E-04 | 7.41E-04 |
| 30_11 | 3.79E-04 | 1.74E-04 | 2.16E-04 | 4.87E-04 | 1.14E-04 | 5.53E-04 | 1.47E-03 |
| 50_6 | 4.95E-04 | 2.69E-04 | 1.33E-04 | 2.02E-04 | 2.09E-04 | 2.06E-04 | 6.57E-04 |
| 50_7 | 5.26E-04 | 6.45E-04 | 2.78E-04 | 5.65E-04 | 3.21E-04 | 3.88E-04 | 1.28E-03 |
| 50_8 | 2.35E-04 | 1.44E-04 | 2.40E-04 | 2.65E-04 | 2.17E-04 | 4.64E-04 | 8.62E-04 |
| 50_9 | 5.21E-04 | 3.58E-04 | 1.06E-04 | 6.36E-04 | 3.12E-04 | 3.37E-04 | 3.64E-04 |
| 50_10 | 5.24E-04 | 4.49E-04 | 2.37E-04 | 3.51E-04 | 4.04E-04 | 3.57E-04 | 6.42E-04 |
| 50_11 | 2.50E-04 | 2.51E-04 | 2.85E-04 | 1.56E-04 | 1.00E-04 | 5.44E-04 | 5.40E-04 |
| 100_6 | 1.57E-04 | 2.75E-04 | 3.46E-04 | 1.24E-03 | 3.39E-04 | 3.61E-04 | 7.00E-04 |
| 100_7 | 3.31E-04 | 1.62E-04 | 1.16E-04 | 4.04E-04 | 2.89E-04 | 2.04E-04 | 6.92E-04 |
| 100_8 | 7.88E-04 | 1.08E-04 | 1.46E-04 | 3.58E-04 | 2.96E-04 | 2.40E-04 | 8.25E-04 |
| 100_9 | 5.78E-04 | 1.69E-04 | 9.63E-05 | 5.53E-04 | 4.37E-04 | 1.57E-04 | 1.08E-03 |
| 100_10 | 5.27E-04 | 3.17E-04 | 1.48E-04 | 1.82E-04 | 2.04E-04 | 4.32E-04 | 3.42E-04 |
| 100_11 | 1.49E-04 | 2.95E-04 | 1.50E-04 | 2.25E-04 | 2.76E-04 | 1.62E-04 | 6.19E-04 |